CONFINED SPACE
CONSTRUCTION PLAN

[Project Name] — Cable Installation in Manhole

[Contractor Name]

Permit No: LOTO-CS-20260822-001

Date: 2026-08-22
Version: 1.0

# Table of Contents

1. Regulatory Basis

2. Project Overview

3. Risk Assessment Report

4. Safety Measures

5. Work Permit

6. Emergency Plan

7. Attachments

8. Compliance Review Report

# 1. Regulatory Basis

## 1.1 Applicable Regulations

| Regulation | Purpose |
| --- | --- |
| Safety Regulations §161-172 | Statutory definition, risk assessment, permit, 17 safety measures |
| Safety Management Manual A.2.5/C.1.8 | Project-level standards, competent person, inspection checklist |
| ISO 31000 | Risk management framework, risk matrix, hazard identification |

## 1.2 Statutory Definition of Confined Space

A confined space is any enclosed space that may present one or more of the following 6 dangers:

1. Explosion (flammable gas/dust exceeds safe limits)

2. Oxygen deficiency (O₂ < 19.5%)

3. Poisoning (toxic gas exceeds exposure limits)

4. Drowning (liquid ingress/accumulation)

5. High temperature (extreme environmental heat)

6. High pressure (residual pressure in pipes/containers)

# 2. Project Overview

## 2.1 Project Information

| Item | Details |
| --- | --- |
| Project Name | [Project Name] — Cable Installation in Manhole |
| Location | [Site Address] |
| Contractor | [Contractor Name] |
| Permit Number | LOTO-CS-20260822-001 |

## 2.2 Confined Space Description

| Parameter | Description |
| --- | --- |
| Space Type | manhole |
| Dimensions | 1.2m x 1.0m x 2.5m |
| Location | Underground utility corridor, near Building A |
| Access Points | Top opening, 600mm diameter |
| Internal Structure | Concrete, vertical ladder |
| Surroundings | Underground utility corridor, near electrical room |

## 2.3 Work Content

| Parameter | Description |
| --- | --- |
| Work Nature | Cable installation and splicing |
| Number of Workers | 3 |
| Duration | 4 hours, 1 shift |
| Materials/Equipment | Power cables, splicing tools, portable lighting |

# 3. Risk Assessment Report

## 3.1 Identified Hazards

| Hazard | Category | Source | Severity | Likelihood | Risk Score | Risk Level |
| --- | --- | --- | --- | --- | --- | --- |
| Hydrogen Sulfide (H₂S > 10 ppm) | atmospheric | Sulfate-reducing bacteria, sewage decomposition | 5 (Catastrophic) | 4 (Likely) | 20 | Extreme |
| Oxygen Deficiency (O₂ < 19.5%) | atmospheric | Biological respiration, oxidation reactions | 5 (Catastrophic) | 4 (Likely) | 20 | Extreme |
| Drowning | physical | Water ingress, liquid accumulation | 5 (Catastrophic) | 3 (Possible) | 15 | Extreme |
| Carbon Monoxide (CO > 25 ppm) | atmospheric | Incomplete combustion, engine exhaust | 5 (Catastrophic) | 3 (Possible) | 15 | Extreme |
| Fall from Height (> 2m) | physical | Deep shafts without fall protection | 4 (Severe) | 3 (Possible) | 12 | High |
| Engulfment / Burial | physical | Material collapse, unstable surroundings | 5 (Catastrophic) | 2 (Unlikely) | 10 | High |

## 3.2 Risk Assessment Method

Risk Score = Severity × Likelihood (ISO 31000 framework)

| Risk Level | Score Range | Action Required |
| --- | --- | --- |
| Extreme | 15-25 | Do not proceed. Must eliminate risk before re-assessment. |
| High | 8-14 | Requires advanced control measures + project manager approval. |
| Medium | 4-7 | Requires control measures + attendant monitoring. |
| Low | 1-3 | Standard PPE + attendant monitoring. |

## 3.3 Ventilation Calculation

| Parameter | Value |
| --- | --- |
| Space Volume | 3.0 m³ |
| Required Airflow | 60.0 m³/h |
| Air Changes per Hour | 20 |
| Minimum Ventilation Time | 30 min |
| Recommended Equipment | Portable axial fan |

# 4. Safety Measures

## 4.1 Engineering Controls

Hazard: H2S

Ventilation ≥ 30 min before entry

Isolate sewage sources (LOTO on valves)

Hazard: Oxygen Deficiency

Mechanical ventilation ≥ 30 min before entry

Continuous ventilation during occupancy

Calculated airflow: volume × 20 air changes/hour

Hazard: Drowning

Isolate water sources (LOTO on valves, blank off pipes)

Dewatering pump on standby

Float switch alarm for water level

Hazard: Co

Eliminate combustion sources near entry (relocate generators)

Ventilation ≥ 30 min before entry

Hazard: Fall

Install tripod + davit at entry point

Guardrail around opening

Secure ladder inside space

Hazard: Engulfment

Shoring/bracing for unstable material

Isolate material feed sources

## 4.2 Administrative Controls

Hazard: H2S

Gas monitoring: H₂S alarm at >5 ppm, evacuate at >10 ppm

No solo entry — attendant required

Emergency rescue plan with SCBA rescue team

Hazard: Oxygen Deficiency

Continuous gas monitoring (O₂ alarm at <20% or >23%)

Attendant stationed outside at all times

Entry permit valid for one shift only

Hazard: Drowning

Attendant monitors for water ingress

Emergency evacuation at first sign of rising water

Hazard: Co

Gas monitoring: CO alarm at >15 ppm, evacuate at >25 ppm

Attendant stationed outside

Hazard: Fall

Fall protection plan for entries > 2m depth

Attendant monitors entrant's harness line

Hazard: Engulfment

No entry if material instability detected

Continuous monitoring of surrounding stability

## 4.3 Personal Protective Equipment (PPE)

Full-body harness

Full-body harness + lifeline

Full-body harness + lifeline for extraction

Gas detector with CO sensor

Gas detector with H₂S sensor on each entrant

Life jacket

Lifeline (rope) connected to tripod

SCBA (Self-Contained Breathing Apparatus) if O₂ < 19.5%

SCBA if CO > 25 ppm

SCBA if H₂S > 10 ppm

Safety helmet

Safety helmet with chin strap

## 4.4 Personnel Requirements

| Role | Requirement | Quantity |
| --- | --- | --- |
| Competent Person | Holds safety qualification certificate, responsible for assessment & approval | 1 |
| Authorized Entrant | Trained, holds valid permit | As needed |
| Attendant (Standby) | Must not multi-task, must be present at all times | 1 |
| Rescue Personnel | Trained, equipped with rescue gear on standby | 1-2 |

# 5. Work Permit

## 5.1 Permit Information

| Item | Details |
| --- | --- |
| Permit Number | LOTO-CS-20260822-001 |
| Work Content | Cable installation and splicing |
| Valid From | 2026-08-22 09:38 |
| Valid Until | (Max one shift) |
| Issuer (Competent Person) | _____________________ |
| Receiver (Authorized Entrant) | _____________________ |

## 5.2 Permit Conditions Checklist (12 items)

☐  1. Risk assessment completed

☐  2. LOTO executed (if applicable)

☐  3. Ventilation ≥ 30 min and gas readings safe

☐  4. All entrants received TBT (Toolbox Talk)

☐  5. Attendant is in position

☐  6. Communication equipment tested and working

☐  7. PPE inspected and donned

☐  8. Rescue equipment in position

☐  9. Explosion-proof lighting in position (if applicable)

☐  10. Entry/exit log prepared

☐  11. Emergency plan communicated to all personnel

☐  12. Work scope confirmed — no scope creep

## 5.3 Statutory Safety Measures (17 items)

☐  1. Confirm space is a confined space

☐  2. Risk assessment completed

☐  3. Work permit issued

☐  4. Competent person designated

☐  5. Workers trained (TBT received)

☐  6. Pre-entry ventilation ≥ 30 min

☐  7. Continuous gas monitoring established

☐  8. Ventilation equipment running continuously

☐  9. LOTO executed (if applicable)

☐  10. Attendant stationed outside

☐  11. Communication equipment in position

☐  12. Emergency rescue equipment in position

☐  13. Rescue personnel on standby

☐  14. PPE equipped and inspected

☐  15. Entry/exit registration system established

☐  16. Explosion-proof lighting in position (if applicable)

☐  17. Post-work: confirm all personnel evacuated

# 6. Emergency Plan

## 6.1 Rescue Equipment

| Equipment | Quantity | Purpose |
| --- | --- | --- |
| Tripod | 1 set | Personnel raising/lowering |
| Winch/Davit | 1 set | 配合 tripod |
| Rescue rope | 2 | Connect to harness |
| Full-body harness | Per person | Each entrant |
| SCBA | 2 sets | Rescue personnel use |
| First aid kit | 1 set | Basic first aid |
| CPR mask | 1 | Artificial respiration |
| Explosion-proof radio | 2 | Internal/external communication |
| Explosion-proof light | 2 | Emergency lighting |

## 6.2 Rescue Procedure

Step 1: Attendant detects abnormality → DO NOT enter the space

Step 2: Immediately notify site supervisor + call emergency number

Step 3: Use tripod + winch to attempt external extraction

Step 4: If external rescue fails → rescue personnel enter with SCBA

Step 5: After extraction, immediately administer first aid / CPR

Step 6: Notify emergency services (999 or local equivalent)

## 6.3 Emergency Contacts

| Service | Number |
| --- | --- |
| Fire/Ambulance | 999 (or local emergency number) |
| Police | 999 (or local emergency number) |
| Labor Bureau | [Local labor bureau number] |
| Project Emergency Contact | [Site emergency number] |

# 7. Attachments

## 7.1 Gas Monitoring Record

| Time | Position | O₂(%) | CO(ppm) | H₂S(ppm) | LEL(%) | Inspector | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Top |  |  |  |  |  |  |
|  | Middle |  |  |  |  |  |  |
|  | Bottom |  |  |  |  |  |  |

## 7.2 Entry/Exit Log

| Name | ID No. | Entry Time | Exit Time | Attendant Confirm | Remarks |
| --- | --- | --- | --- | --- | --- |

## 7.3 LOTO Record

| Energy Type | Equipment/Valve ID | Lock ID | Locked By | Lock Time | Unlock Time | Confirmed By |
| --- | --- | --- | --- | --- | --- | --- |

## 7.4 TBT Training Record

| Name | Position | Training Content | Date | Signature |
| --- | --- | --- | --- | --- |

# 8. Compliance Review Report

## 8.1 Summary

| Item | Result |
| --- | --- |
| Total Check Items | 49 |
| Passed | 44 |
| Failed | 5 |
| Pass Rate | 44/49 (89.8%) |
| Overall Status | CONDITIONAL PASS |

## 8.2 Section Results

### §161 — Risk Assessment Report

| # | Check Item | Status |
| --- | --- | --- |
| 1 | Work location description complete (type/dimensions/location/access) | PASS |
| 2 | All 4 hazard dimensions identified (atmospheric/physical/chemical/structural) | PASS |
| 3 | Risk assessment method clear (matrix formula) | PASS |
| 4 | Risk levels correctly determined | PASS |
| 5 | Control measures cover all identified hazards | PASS |
| 6 | Emergency plan includes rescue procedures and contacts | FAIL |
| 7 | Assessor signature field complete | PASS |

### §163 — Work Permit

| # | Check Item | Status |
| --- | --- | --- |
| 1 | Work content and scope clearly defined | PASS |
| 2 | Validity period reasonable (max one shift) | PASS |
| 3 | Permit conditions confirmed item-by-item (12 items) | PASS |
| 4 | Issuer is a competent person | PASS |
| 5 | Receiver is a designated person | PASS |

### §165 — Safety Measures (17 items)

| # | Check Item | Status |
| --- | --- | --- |
| 1 | Space confirmed as confined space | PASS |
| 2 | Risk assessment completed | PASS |
| 3 | Work permit issued | PASS |
| 4 | Competent person designated | PASS |
| 5 | Workers trained (TBT received) | PASS |
| 6 | Pre-entry ventilation ≥ 30 min | PASS |
| 7 | Continuous gas monitoring established | PASS |
| 8 | Ventilation equipment running continuously | PASS |
| 9 | LOTO executed (if applicable) | PASS |
| 10 | Attendant stationed outside | PASS |
| 11 | Communication equipment in position | PASS |
| 12 | Emergency rescue equipment in position | FAIL |
| 13 | Rescue personnel on standby | FAIL |
| 14 | PPE equipped and inspected | PASS |
| 15 | Entry/exit registration system established | PASS |
| 16 | Explosion-proof lighting in position (if applicable) | PASS |
| 17 | Post-work: all personnel evacuated confirmation | PASS |

### §166-167 — Personnel Qualification

| # | Check Item | Status |
| --- | --- | --- |
| 1 | Competent person qualification certificate valid | PASS |
| 2 | Entrant training records complete | PASS |
| 3 | Attendant duties clearly communicated | PASS |

### §168-172 — Underground Work (if applicable)

| # | Check Item | Status |
| --- | --- | --- |
| 1 | Earth support design approved | PASS |
| 2 | Groundwater control measures in place | PASS |
| 3 | Adjacent structure protection assessed | PASS |
| 4 | Enhanced ventilation meets requirements | PASS |

### Safety Management Manual A.2.5

| # | Check Item | Status |
| --- | --- | --- |
| 1 | Risk assessment complete | PASS |
| 2 | Safety measures specific and actionable | PASS |
| 3 | Emergency plan feasible | PASS |
| 4 | Personnel training plan included | PASS |
| 5 | Supervision system clearly defined | PASS |

### Safety Management Manual C.1.8

| # | Check Item | Status |
| --- | --- | --- |
| 1 | Attendant present and not multitasking | PASS |
| 2 | Continuous ventilation operational | PASS |
| 3 | Gas monitoring continuous | PASS |
| 4 | O₂ in 19.5-23.5% range | PASS |
| 5 | Flammable gas < 10% LEL | PASS |
| 6 | Toxic gases within limits | PASS |
| 7 | Communication working | FAIL |
| 8 | Rescue equipment in position | FAIL |

## 8.3 Improvement Suggestions

§161 — Risk Assessment Report

Emergency plan includes rescue procedures and contacts

§165 — Safety Measures (17 items)

Emergency rescue equipment in position

Rescue personnel on standby

Safety Management Manual C.1.8

Communication working

Rescue equipment in position